THIS PROGRAM NEEDS VISUAL BASIC 3.0

====================================

This file provides a brief instruccion to permet a rapid understanding

of how Espectrum 1.1 works.

FIRST WINDOW

OPEN FILES:

Select the format of file you want, click on the file and then

click  on the "Obrir" button.  The spectrum will be plotted in

a little window.

Extension:  .cal = ASCII files

.bin = Binary files

For the analysis of the spectrum just click on the VISOR button

and you will be in the visor window.

CALIBRATE:

Click on the CALIBRAT button and then introduce the parameters of

the calibrate in energy.  Click on the OK button.

VISOR WINDOW

ZOOM Button:

Click on the "magnifying glass man" button, then select by clicking

the first and last channel of then zone you want to amplify.

In cursor mode put the cursor on te selected channel and then click

on the spectrum with the left button of the mouse.

ROI Button:

With this you can calculate the pondered mean "Centroide" and the

standard deviation*2.35 (FWHM) from a distribution.  Just select the

starting and ending channel of the spectrum to get a correct fit to

the data.

CLIPBOARD Button:

When clicking the spectrum is sent to the clipboard

DOOR Button:

Drive you to the first window of the program

RUBBER Button:

When you click on it a squared rubber appear.  Click on this rubber

and it follows the mouse pointer rubbing the noise plotted before.

Click again on the rubber and it will disapear.

M E N U :

ESCALA:

Change the scale.

NOISE:

With this you can draw the noise function:

GRAU 1: polynomial of degree 1.

GRAU 1(MIN2): polynomial of degree 1 calculated by taking 75

preceding points to the starting point and 75 after the ending

point, then adjusting by least squares.

GRAU 2: polynomial of degree 2.

GRAU 3: polynomial of degree 3.

AJUSTAR GAUSSIANA: fits a gaussian function plus a "noise" po-

lynomial by numeric method. This option is only avai-

lable in the complete version of this program.

When you select de degree n of the noise polynomial, you must click

on the spectrum on n+1 points necessary to calculate the parameters

of the polynomial.  Then the noise is plotted.  If you don't like

this plot you can rub it with the rubberbutton and try again.

If you select the option GRAU 1(MIN2) you haven't to choose any point.

You are ready for the fit.

If you have any suggestion please write to me to the following address:

Francisco J. Fabra Ortiz

C\Corbella, 6

Carlet  46240

Valencia

SPAIN